Tasks book

Technical data:

–	Operation depth: up to 2000 m

–	Operation temperature: at least 0°C to 35°C

–	Power: Lithium batteries

–	Telemetry: preferably Iridium

–	Operating life: minimum 3 years

–	Activation: magnetic switch

–	Data storage: minimum 128 MB

–	GPS

Sensors:

–	CTD	–	Salinity: minimum range = 0-30 psu, initial accuracy = ±0.003 psu, drift = <0.01 psu/ 5 years

–	Temperature: minimum range = 0°C – 30°C, initial accuracy = ±0.002°C, drift = <0.002°C/5 years

–	Pressure: range = 0 dbar-2100 dbar, initial accuracy = <±2.5 dbar, drift = <5 dbar/5 years

–	Oxygen	–	optode: range: 0-500 µM/l, resolution: <1 µM/l, accuracy: <8 µM/l or 5%

–	Chlorophyll	–	range: 0-30 µg/l, sensitivity: < 0.02 µg/l

Guarantee: minimum 1 year

Delivery: National Institute of Marine Geology and Geoecology, 23-25 Dimitrie Onciul street, Bucharest, RO-024053